# Project Pitch: Smart Traffic Control

Name: Ziquan Sun

Application domain: Smart City

Hardware Platform: ESP32

# Executive Summary

The project aims to build a smart traffic control system that makes traffic light and speed limit decisions by sensing its environment and gathering relevant information from the internet. It is especially helpful in high speed and high traffic intersections with heavy pedestrian flows, such as some small-town high street or popular tourist destinations.

# System Architecture

The system is built based on a sensing-processing-actuation logic with modular design and simple adjustability so it can be easily applied to different local traffic planning needs.

## Sensing

- DHT22 Sensor: Captures ambient temperature and humidity data.

- IR Sensor: Detects pedestrian presence for automated traffic light changes.

- Ultrasonic Distance Sensor: Detects vehicle for the control system to make decisions, also for speed warning.

## Processing

The processing is based on an ESP32.

## Actuation

- RGB LED: A set of LED to simulate traffic light.

- LED Matrix: A low-definition display for showing dynamic speed sign and warnings to road users.

- Servo Motor: A simulation for possible pedestrian crossing boom gate.

# Behavioural Logic

| State | Trigger | Output |
| --- | --- | --- |
| Standard State | Boot | Traffic light flip with normal interval. |
| Pedestrian Crossing | Pedestrian Detected | Interrupt standard state to allow pedestrian to cross. Operate servo motor to open boom gate. |
| Wet | Road might be wet based on humidity detection along with weather api | Reduced speed limit on dynamic speed sign |
| Ice | Possible frozen surface based on temperature and humidity detection along with weather api | Reduced speed limit on dynamic speed sign and print warning message. |
| Traffic Congestion | Multiple vehicle queue up detected | Reduced speed limit on dynamic speed sign and reduced traffic light interval |

# Embedded Intelligence

- Non-Blocking: The normal traffic light runs in the background by utilising millis() timer, so it will not be interrupted by other processing activities.

- IoT Enabled: The system connects to the Internet to gather information about weather and traffic.

# Cloud Integration

The system will upload sensor readings and running status to the cloud to display and monitor changes.